Flight Report

4 September 2006

Two main objectives to this flight. Determine airspeed indicator accuracy and stall speed.

I conducted three separate runs at 6000 feet to collect the data. I set the altimeter to 29.92. Temp on the ground was 17 degrees C at takeoff.  One run at 2800 rpm one at 3000 rpm, one at 3200.  It actually takes quite a while to ensure the airplane is smoothed out a running straight and level before recording the data. I used a excel spread sheet and recorded the gps track, ground speed and Indicated airspeed at three different directions.  I inputted the data into the excel spreadsheet and my airspeed accuracy is good.

| Vtrue | Engine | Indicated | Alt |
| --- | --- | --- | --- |
| (mph) | RPM | Airspeed | feet |
| 110.0 | 2800 | 105 | 6000 |
| 115.9 | 3000 | 112 | 6000 |
| 123.0 | 3200 | 120 | 6000 |

I’ll do additional flight testing at different altitudes to see what other results I receive.

I did a power off stall series and my stall speed is 50 mph IAS.

At this point the fuel gage was showing ¼ of a tank so I took her home. Another good landing in a slight left crosswind. No problems landing my airplane yet.  My hydraulic brakes are not as good as I am used to on the Cessna’s I have flown where you can almost stand on a dime when stopping.  These are the Great Planes brakes and while ok, I take much longer to slow down than I am used to once planted on the ground.

All engine temps green. Highest EGT I saw was 1300 on climb out.  CHT 325, Oil temp 160 or below. Oil Press 70 psi, I’m hoping that will come down a little soon as the engine breaks in more.  Post flight inspection, I saw a dribble of oil at the front prop seal. I’ll watch that one for a while. Also a dribble at the back end I could not place.