| Slow Speed Taxi Test Card |  |
| --- | --- |
| 1 | Confirm weight and balance calculations, CG within limits |
| 2 | Confirm 10 gallons of fuel on board |
| 3 | Execute Preflight Inspection Checklist |
| 4 | Execute Before Engine Start Checklist |
| 5 | Execute Engine Start Checklist |
| 6 | Record engine parameters at idle power 1 minute after start |
| 7 | Execute Before Taxi Checklist |
| 8 | Record engine parameters at idle power 10 minutes after start |
| 9 | Taxi straight ahead, verify neutral steering wheel position |
| 10 | Smoothly apply brakes, verify even braking action |
| 11 | Execute 90 degree left turn, observe performance and flight instrument operation |
| 12 | Execute 90 degree right turn, compare with left turn performance |
| 13 | Execute 180 degree left turn, observe performance |
| 14 | Execute 180 degree right turn, compare with left turn performance |
| 15 | Execute 360 degree left turn, measure and record turn radius |
| 16 | Execute 360 degree right turn, measure and record radius |
| 17 | Execute 360 degree left turn with brakes, record radius |
| 18 | Execute 360 degree right turn with brakes, record radius |
| 19 | Record engine parameters at idle |
| 20 | Execute slow speed S-turns, verify smooth and even wheel steering |
| 21 | Practice slow speed taxi maneuvers until comfortable |
| 22 | Execute After Landing, Before Engine Shut Down, and Engine Shut Down Checklists. |
| 23 | Execute Post Flight Inspection Checklist |
| 24 | Thoroughly inspect engine plumbing for leaks |
| 24 | Thoroughly inspect hydraulic plumbing for leaks |
| 26 | Thoroughly inspect landing gear |
| 27 | Thoroughly inspect propeller |
| 28 | Examine fuel system screens and filters for metal |
| 29 | Examine oil system screens and filters for metal |
| 30 | Fix all squawks before continuing test program. |

| High Speed Taxi Test Card 1 |  |
| --- | --- |
| 1 | Confirm weight and balance calculations, CG within limits |
| 2 | Confirm 10 gallons of fuel on board |
| 3 | Confirm “First Flight Test Card” is available in cockpit |
| 4 | Execute Preflight Inspection Checklist |
| 5 | Execute Before Engine Start Checklist |
| 6 | Execute Engine Start Checklist |
| 7 | Record engine parameters at idle power 1 minute after start |
| 8 | Execute Before Taxi Checklist |
| 9 | Record engine parameters at idle power 10 minutes after start |
| 10 | Perform slow speed taxi maneuvers to reaffirm operation of steering and brakes |
| 11 | Conduct High Speed Tests. Start RPMs at 1800, Increase speed to 40 mph (80% of Stall speed). At 40 mph close throttle. Repeat test with +200 rpm until use of full throttle. Observe controllability throughout operation. |
| 12 | Attempt to raise the nose starting at 35 mph |
| 13 | Record speed at which nose comes up ________ |
| 14 | Attempt to rock the wings SLIGHTLY starting at 35 mph with the nose on the ground |
| 15 | Execute heavy braking (incrementally increased) |
| 16 | Record engine parameters at idle power |
| 17 | Determine distance required for high speed abort |
| 18 |  Accelerate to nose wheel lift off speed and maintain till preplanned execution point |
| 19 |  Chop throttle to idle and apply heavy braking to bring aircraft to a complete stop |
| 20 |  Mark distance from execution point to stop point |
| 21 |  Repeat test at least 3 times and measure worst case (longest) distance ________ |
| 22 | Execute After Landing, Before Engine Shut Down, and Engine Shut Down Checklists. |
| 23 | Thoroughly inspect engine plumbing for leaks |
| 24 | Thoroughly inspect hydraulic plumbing for leaks |
| 25 | Thoroughly inspect landing gear |
| 26 | Thoroughly inspect propeller |
| 26 | Calculate takeoff speed – Nose lift speed X 1.25 = _________ |
| 28 | Calculate High Speed Abort Distance |
| 29 |  Max stop distance X 1.3 = ____________ |
| 30 | Fix all squawks before continuing test program |

| High Speed Taxi Test Card 2 |  |
| --- | --- |
| 1 | Confirm weight and balance calculations, CG within limits |
| 2 | Confirm 10 gallons of fuel on board |
| 3 | Confirm “First Flight Test Card” is available in cockpit |
| 4 | Execute Preflight Inspection Checklist |
| 5 | Execute Before Engine Start Checklist |
| 6 | Execute Engine Start Checklist |
| 7 | Record engine parameters at idle power 1 minute after start |
| 8 | Execute Before Taxi Checklist |
| 9 | Record engine parameters at idle power 10 minutes after start |
| 10 | Perform slow speed taxi maneuvers to reaffirm operation of steering and brakes |
| 11 | Conduct High Speed Tests. Start RPMs at 1800, Increase speed to 65 mph (1.3 of Stall speed). At 65 mph close throttle. Repeat test with +200 rpm until at full throttle. Observe controllability throughout operation. I |
| 12 | Conduct Landbacks. At 65 mph close throttle, lift nose and allow airplane to lift off into ground effect and then settle back onto runway. |
| 13 | Observe rudder effectiveness and aileron effectiveness during landback |
| 14 | Attempt to rock the wings SLIGHTLY during landback. |
| 15 | Execute heavy braking (incrementally increased) |
| 16 | Record engine parameters at idle power |
| 17 | Execute After Landing, Before Engine Shut Down, and Engine Shut Down Checklists. |
| 18 | Execute Post Flight Inspection Checklist |
| 19 | Thoroughly inspect engine plumbing for leaks |
| 20 | Thoroughly inspect hydraulic plumbing for leaks |
| 21 | Thoroughly inspect landing gear |
| 22 | Thoroughly inspect propeller |
| 23 | Fix all squawks before continuing test program |

| First Flight Test Card |  |  |
| --- | --- | --- |
| 1 |  | Confirm 12 gallons of fuel on board (4 times hourly consumption rate) |
| 2 |  | Confirm CG will remain in the forward half of the CG range for the entire flight |
| 3 |  | Execute Preflight Inspection Checklist |
| 4 |  | Execute Before Engine Start Checklist |
| 5 |  | Execute Engine Start Checklist |
| 6 |  | Record engine parameters 1 minute after engine start |
| 7 |  | Execute Before Taxi Checklist |
| 8 |  | Verify operation of all aircraft systems: |
| 9 |  |  Engine instruments and engine operation |
| 10 |  |  Flight controls |
| 11 |  |  Brakes |
| 12 |  |  Flight instruments |
| 13 |  |  Avionics |
| 14 |  | Record engine parameters 10 minutes after engine start |
| 15 |  | Call for taxi, inform tower/Unicom that an experimental aircraft is on it’s first test flight |
| 16 |  | Execute slow speed S-turns during taxi to confirm steering |
| 17 |  | Execute Before Take Off Checklist |
| 18 |  | Record run-up engine parameters |
| 19 |  | Call for clearance, line up on runway centerline, and execute first flight take off procedure |
| 20 |  | Climb above airport to 3000 feet AGL level flight (4000 ft) using shallow turns. |
| 21 |  | Slowly reduce power to maintain airspeed at 90 mph |
| Flight Control Checks |  |  |
| 22 |  | Yaw aircraft 5 degrees left and right and observe aircraft response |
| 23 |  | Raise nose 3 degrees, stabilize, and observe aircraft response |
| 24 |  | Level off |
| 25 |  | Lower nose 3 degrees, stabilize, and observe aircraft response |
| 26 |  | Level off |
| 27 |  | Bank aircraft 5 degrees left, then 5 degrees right, observe aircraft response |
| 28 |  | Execute clearing turns at no more than 10 degrees of bank |
| 29 |  | Execute two 360 degree turns at 20 degrees of bank, one in each direction, and observe aircraft response |
| 30 |  | Climb above airport to level flight at 5000 feet AGL (6000 feet) |
| Simulate Traffic Pattern |  |  |
| 31 |  | Establish pattern airspeed (80-90 mph) |
| 32 |  | Execute Descent and Before Landing Checklists |
| 33 |  | Establish an approach speed of 75 mph |
| 34 |  | Practice an approach to landing at 4000 feet AGL (5000 feet) |
| 35 |  | Practice an approach to landing at 3000 feet AGL (4000 feet) |
| 36 |  | Clean up aircraft, execute Cruise Climb Checklist, and climb back to 5000 feet AGL (6000 feet) |
| 37 |  | Execute an approach to stall maneuver |
| 38 |  |  Execute clearing turns  Stabilize altitude, airspeed, and heading  Apply carb heat  Establish landing configuration  Reduce power to 1000 RPM and trim  Raise nose as required to maintain altitude, note control inputs required to maintain level coordinated flight  Discontinue maneuver at onset of buffet, recover and record the reference pre-stall speed _________ |
| 39 |  | Practice recoveries from approach to stall until comfortable |
| 40 |  | Establish and maintain level flight and trim |
| 41 |  | Calculate final approach speed – ref speed X 1.3 = ________ |
| 42 |  | Execute Descent and Before Landing Checklists |
| 43 |  | Execute low approaches as required |
| 44 |  | Land with a planned touchdown point within the first 1000 feet of the runway |
| 45 |  | Execute After Landing, Before Engine Shut Down, and Engine Shut Down Checklists |
| 46 |  | Execute Post Flight Inspection Checklist |
| Post Flight Data Collection (from video/audio/computer) |  |  |
|  | Record engine performance parameters during take off roll |  |
|  | Record engine performance parameters during climb |  |
|  | Record engine performance parameters during cruise at 3000’ |  |
|  | Record engine performance parameters during cruise at 5000’ |  |
|  | Record engine performance parameters during descent |  |
|  | Record reference stall speed in Flight Manual |  |
|  | Record any pertinent comments (notes, cautions, warnings) regarding aircraft handling in the Flight Manual |  |
| Post Flight Actions |  |  |
|  | Engine cooling changes |  |
|  | Flight control rigging changes |  |
|  | Aileron trim changes |  |
|  | Rudder trim changes |  |
|  | Evaluate propeller performance/suitability, change if required |  |
|  | Repair/Replace any damaged components |  |
|  | Perform thorough Airworthiness Inspection, concentrating on: |  |
|  |  Engine/fuel plumbing leaks |  |
|  |  Hydraulic plumbing leaks |  |
|  |  Landing gear and brakes |  |
|  |  Flight control cables and attach points |  |
|  |  Propeller bolt torque |  |
|  |  Engine mount bolt torque |  |
|  | Pitot-Static system modifications |  |

| \Flight Parameter\Phase | 1 min after start | 10 min after start | Run-up | Take Off | Climb | Cruise | Descent |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Altitude |  |  |  |  |  |  |  |
| Airspeed |  |  |  |  |  |  |  |
| OAT |  |  |  |  |  |  |  |
| RPM |  |  |  |  |  |  |  |
| Oil Pressure |  |  |  |  |  |  |  |
| EGT |  |  |  |  |  |  |  |
| CHT |  |  |  |  |  |  |  |
| Oil Temperature |  |  |  |  |  |  |  |