| Flight #: 3 – VALIDATE ENGINE RELIABILITY |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Validate that actions affecting engine operation function properly |  |  |  |
| Check | Action |  |  |
|  | # DEPARTURE |  |  |
|  | Do not change throttle settings, mixture, or fuel tanks |  |  |
|  | Remain above the airport |  |  |
|  | Climb out at 90 MPH |  |  |
|  | Climb to 5000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3400 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Check hands off level flight for ground trim tab adjustments |  |  |
|  | Apply carb heat and note changes |  |  |
|  | Lean engine and note changes |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | Record engine pressures and temperatures |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | Pull cowlings and inspect engine carefully |
|  | Inspect airframe carefully |

NOTES

| Flight #: 4 – SLOW FLIGHT TEST |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Become familiar with slow flight handling characteristics |  |  |  |
| Check | Action |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 90 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3000 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | Perform 2 clearing turns |  |  |
|  | Slow to 65 MPH |  |  |
|  | Feel and maintain altitude |  |  |
|  | 360 turn left, then 360 right, shallow bank |  |  |
|  | Check CHTs & Oil Temp |  |  |
|  | Slow to 60 MPH |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # Maintain Altitude |
|  | # 360 turn left, 360 turn right, shallow bank |
|  | Check CHTs and Oil Temp |
|  | Slow to 60 MPH |
|  | Maintain altitude |
|  | 360 turn left, 360 turn right, shallow bank |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 5 – CLIMBS AND DESCENTS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Monitor engine performance during climbs and descents |  |  |  |
| Check | Action |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 90 MPH |  |  |
|  | Climb to 2000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | 110 MPH climb for two minutes – full power |  |  |
|  | Record engine temperatures and pressures – and OAT |  |  |
|  | Stabilize temperatures |  |  |
|  | 100 MPH climb for two minutes – full power |  |  |
|  | Record engine temperatures and pressures – and OAT |  |  |
|  | Stabilize temperatures |  |  |

NOTES

To obtain this test flight data you must use common sense.  Don't cook your engine!  Don't shock cool your engine!  Elements of this testing might be done in conjunction with other test flights during several different flights to avoid overheating or shock cooling your engine.

| Check | Action |
| --- | --- |
|  | # Moderate power descent to 2000 ‘ – do not exceed 180 MPH |
|  | 90 MPH climb for two minutes – full power |
|  | Record engine temperatures and pressures – and OAT |
|  | Stabilize temperatures |
|  | 80 MPH climb for two minutes – full power |
|  | Record engine temperatures and pressures – and OAT |
|  | Stabilize temperatures |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 6 – AIRSPEED IN-FLIGHT ACCURACY CHECK |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine accuracy of the airspeed indicator throughout a range of airspeeds |  |  |  |
| Check | Action |  |  |
|  | Determine altitudes at which you desire airspeed data. |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 90 MPH |  |  |
|  | Climb to 2000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Check for Trim hands off level flight for ground adjustment |  |  |
|  | Set Mixture |  |  |

NOTES

When you fly the three ground tracks for each power setting the IAS should be exactly the same.  If not, make sure you're at the exact same altitude and just be patient --- sometimes it takes a few minutes for the airspeed to settle after a turn and a few burbbles.

| Check | Action |
| --- | --- |
|  | # 2400 RPM, constant altitude - 5500 MSL |
|  | - Record OAT |
|  | - Fly ground track 360 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 180 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 90 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Record ground track from GPS |

NOTES

Altitude:	5500 MSL

Throttle:	2400 RPM

|  | 360 | 180 | 90 |
| --- | --- | --- | --- |
| OAT |  |  |  |
| IAS |  |  |  |
| Ground Speed |  |  |  |

| Check | Action |
| --- | --- |
|  | # Set power 2800 RPM, constant altitude - 5500 MSL |
|  | - Reset Mixture |
|  | - Record MAP |
|  | - Fly ground track 360 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 180 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 90 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |

NOTES

Altitude:	5500 MSL

Throttle:	2800 RPM

|  | 360 | 180 | 90 |
| --- | --- | --- | --- |
| IAS |  |  |  |
| Ground Speed |  |  |  |

| Check | Action |
| --- | --- |
|  | # Set power 3000 RPM, constant altitude - 5500 MSL |
|  | - Reset Mixture |
|  | - Record MAP |
|  | - Fly ground track 360 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 180 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 90 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |

NOTES

Altitude:	5500 MSL

Throttle:	3000 RPM

|  | 360 | 120 | 240 |
| --- | --- | --- | --- |
| IAS |  |  |  |
| Ground Speed |  |  |  |

| Check | Action |
| --- | --- |
|  | # Set power 3200 RPM, constant altitude - 5500 MSL |
|  | - Reset Mixture |
|  | - Record MAP |
|  | - Fly ground track 360 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 180 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |
|  | - Fly ground track 90 |
|  | - Record IAS from airspeed indicator |
|  | - Record ground speed from GPS |

NOTES

Altitude:	5500 MSL

Throttle:	3200 RPM

|  | 360 | 120 | 240 |
| --- | --- | --- | --- |
| IAS |  |  |  |
| Ground Speed |  |  |  |

| Check | Action |
| --- | --- |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | Calculate True Airspeeds using the attached Excel Spreadsheet (True Airspeed Calculator) |
|  | Update Aircraft Operations Manual |

NOTES

By using the attached Excel Spreadsheet, your ground track does not have to be exactly on the cardinal heading.  If it isn't, record the ground track you did have on the spreadsheet.

What you're testing is the accuracy of your airspeed indicator.  Consider doing this test at close to stall speeds w/ & w/out flaps to get an idea of your TAS for stall speed.

| Flight #: 7 – STALLS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine actual stall speeds in landing and takeoff configuration |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Fill fuel tanks to full |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 90 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Check hands off level flight for trim adjustment |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # POWER OFF STALLS |
|  | No flaps |
|  | Slowly decelerate while maintaining altitude |
|  | Keep ball centered with rudder |
|  | Note stall speed |
|  | Recover altitude and speed |
|  | Slow down to 100 MPH |
|  | Slowly decelerate while maintaining altitude |
|  | Keep ball centered with rudder |
|  | Note stall speed |
|  | Recover altitude and speed |

NOTES

SPEED IAS (MPH)

POWER-OFF STALL, NO FLAP:

| Check | Action |
| --- | --- |
|  | # POWER ON STALLS |
|  | Set power to 2200 RPM |
|  | Slowly pull back elevator |
|  | Keep ball centered with rudder |
|  | Note stall speed |
|  | Recover altitude and speed |

NOTES

SPEED IAS (MPH)

POWER-ON STALL, 2200 RPM, NO FLAP:

| Check | Action |
| --- | --- |
|  | Full power |
|  | Slowly pull back elevator |
|  | Keep ball centered with rudder |
|  | Note stall speed |
|  | Recover altitude and speed |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | Update POH with actual stall speeds |

NOTES

SPEED IAS (MPH)

POWER-ON STALL, FULL POWER:

| Flight #: 8 – CLIMB SPEEDS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Establish best rate of climb speed (Vy) - Establish best angle of climb speed (Vx) - Establish best glide rates - Learn to visualize power-off glide descent rate |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Bring small stopwatch/timer |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 90 MPH |  |  |
|  | Climb to 2500’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight, Full rich mixture |  |  |
|  | # Do 2 clearing turns |  |  |

NOTES

As mentioned before, use common sense.  Don't cook your engine and don't shock cool your engine.  These tests might be best done over several flights in conjunction with other tests.

| Check | Action |
| --- | --- |
|  | # CLIMB TEST#1 |
|  | Establish 120 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 3000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | Trim Level, Cool Engine |
|  | GLIDE TEST #1 |
|  | Descend at 120 MPH - Trim hands off |
|  | Record descent rate from VSI |
|  | Perform a 90 degree turn @ 15 deg. bank and record altitude lost |
|  | Perform 180 degree turn @ 15 deg. bank and record altitude lost |
|  | Perform 360 degree turn @ 15 deg. bank and record altitude lost |
|  | CLIMB TEST#2 |
|  | Establish 110 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 3000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | # Trim Level, Cool Engine |
|  | Descend at 110 MPH, Trim hands off, Record Rate |
|  | Perform & record altitude lost in 90, 180 & 360 turns |

NOTES

| IAS | Climbed to: | Climb Rate (FPM) | Descent Rate (FPM) | 90 Turn | 180 Turn | 360 Turn |
| --- | --- | --- | --- | --- | --- | --- |
| 120 |  |  |  |  |  |  |
| 110 |  |  |  |  |  |  |

| Check | Action |
| --- | --- |
|  | # CLIMB TEST#3 |
|  | Establish 100 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 3000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | # Descend to 2500 MSL |
|  | Trim Level, Cool Engine |
|  | Descend at 100 MPH, Trim hands off, Record Rate |
|  | Perform & record altitude lost in 90, 180 & 360 turns |
|  | CLIMB TEST#4 |
|  | Establish 90 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 2000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | # Descend to 2500 MSL |
|  | Trim Level, Cool Engine |
|  | Descend at 90 MPH, Trim hands off, Record Rate |
|  | Perform & record altitude lost in 90, 180 & 360 turns |

NOTES

| IAS | Climbed to: | FPM |
| --- | --- | --- |
| 100 |  |  |
| 90 |  |  |

| Check | Action |
| --- | --- |
|  | # CLIMB TEST#5 |
|  | Establish 80 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 3000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | # Descend to 2500 MSL |
|  | Trim Level, Cool Engine |
|  | Descend at 80 MPH, Trim hands off, Record Rate |
|  | Perform & record altitude lost in 90, 180 & 360 turns |
|  | CLIMB TEST#6 |
|  | Establish 70 MPH climb - Trim hands off |
|  | Begin 1 minute timer as we pass thru 3000 MSL |
|  | At end of 1 minute, record altitude |
|  | Ending Altitude= FPM= |
|  | # Descend to 2500 MSL |
|  | Trim Level, Cool Engine |
|  | Descend at 70 MPH, Trim hands off, Record Rate |
|  | Perform & record altitude lost in 90, 180 & 360 turns |

NOTES

| IAS | Climbed to: | FPM |
| --- | --- | --- |
| 80 |  |  |
| 70 |  |  |

| Check | Action |
| --- | --- |
|  | LANDING |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and "Grin" |
|  | POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |
|  | Use graph to compute Vy and Vx |
|  | Use graph to compute best glide speed |

NOTES

| Flight #: 9 – STABILITY AND CONTROL CHECKS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Determine longitudinal stability - Determine lateral-directional stability - Determine spiral stability |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | These tests cannot be accomplished until any necessary trim tabs have been installed so the aircraft can be flown hands off |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | LONGITUDINAL STABILITY TEST |
|  | # Record airspeed at 2800 RPM (Airspeed ___ = A) |
|  | Lightly pull on stick to reduce airspeed by 10% |
|  | New airspeed: A-10%= ___ = B |
|  | Does the acft. require continued pull force to maintain the new airspeed? |
|  | If yes: Pull stick to reduce airspeed to A-20% =___= C |
|  | # Does acft. require still more pull force to maintain airspeed C? |
|  | If yes: POSITIVE static stability |
|  | If no to either B or C airspeed, NEUTRAL static stability |
|  | If requires a push force for B or C airspeeds, then NEGATIVE static stability |
|  | ---- repeat test using a PUSH test instead of PULL test |
|  | TEST FOR POSITIVE DYNAMIC LONGITUDINAL STABILITY (SHORT PERIOD) |
|  | Trim for cruise @ 2800 RPM |
|  | Push nose down 5 degrees, then up to level attitude |
|  | As attitude reaches level, release stick |
|  | # If briefly oscillates about the trim attitude before settling at trim attitude then POSITIVE DYNAMIC LONGITUDINAL STABILITY (SHORT PERIOD) |

NOTES

| Check | Action |
| --- | --- |
|  | TEST FOR POSITIVE DYNAMIC LONGITUDINAL STABILITY (LONG PERIOD) |
|  | # Trim for cruise @ 2800 RPM, Record Airspeed ___ = A |
|  | Pull stick for A - 5 MPH and release stick |
|  | Expect to oscillate about the trim speed before in dampens out |
|  | If amplitude INCREASES with time = NEGATIVE DLS |
|  | If amplitude CONTINUES to oscillate = NEUTRAL DLS |
|  | # If returns to cruise trim & speed = POSITIVE DLS |
|  | TEST FOR LATERAL/DIRECTIONAL CONTROL STABILITY |
|  | Set low cruise speed (BELOW MANEUVERING SPEED) & trim |
|  | Slowly enter a sideslip until either full rudder or full aileron deflection |
|  | # Release aileron while holding full rudder |
|  | ---> low wing should raise to level |
|  | TEST FOR STATIC DIRECTIONAL STABILITY |
|  | Set low cruise speed (BELOW MANEUVERING SPEED) & trim |
|  | # Slowly yaw with rudder while keeping acft level with aileron ----- release rudder |
|  | Should return to no yaw condition |

NOTES

| Check | Action |
| --- | --- |
|  | TEST SPIRAL STABILITY |
|  | # (This will demonstrate the aircraft's tendency to raise the low wing when controls are released in a bank) |
|  | Bank 15 to 20 degrees and release controls |
|  | ---> If bank angle DECREASES = POSITIVE SS |
|  | ---> If bank angle STAYS THE SAME = NEUTRAL SS |
|  | ---> If bank angle INCREASES = NEGATIVE SS |
|  | # LANDING |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 10 – ACCELERATED STALLS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Further explore stall characteristics of the aircraft |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Consider wearing a parachute & practice egress |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 10,000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit Airspeed to MANEUVERING SPEED |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # ACCELERATED STALL TEST |
|  | Hold 15 degrees bank and slow the aircraft until stall |
|  | ---> Airspeed at stall with 15 degrees bank = ___ |
|  | ---> Airspeed at stall with 30 degrees bank = ___ |
|  | ---> Airspeed at stall with 45 degrees bank = ___ |
|  | ---> Airspeed at stall with 60 degrees bank = ___ (2g) |
|  | # LANDING |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 11 – "G" LIMIT TESTING |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: Ensure aircraft meets the "G" limit capabilities |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Ensure Weight & Balance is within Aerobatic limits |  |  |
|  | # Consider wearing parachute and practice egress |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 10,000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | # Make 2 clearing turns |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | # Establish 30 degree bank & pull on stick to achieve 2 G |
|  | Release pressure & fly straight & level |
|  | # Establish 30 degree bank & pull on stick to achieve 3 G |
|  | Release pressure & fly straight & level |
|  | # Establish 30 degree bank & pull on stick to achieve 4 G |
|  | Release pressure & fly straight & level |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 12a – EXPLORE WEIGHT & BALANCE LIMITS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: Determine affect of change to aft balance and progressively increasing weights to establish maximum weight - Pilot (me) plus 80 lb. passenger & max. fuel |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Carefully weigh and secure ballast |  |  |
|  | # Compute & record new weight & balance |  |  |
|  | # DEPARTURE |  |  |
|  | Note: 10% flaps can help lift tail as aft weight is added. |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | Record climb performance: FPM = ____ |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | # SLOW FLIGHT & STALLS |  |  |
|  | 360 turns, slow flight & power off stall, recover |  |  |
|  | Execute power-on stalls @ 2600 RPM |  |  |

NOTES

Salt bags work well as ballast.  Make sure they are securely belted in.

| Check | Action |
| --- | --- |
|  | STABILITY & CONTROL CHECKS |
|  | # Longitudinal Stability: Record Airspeed @ 2800 RPM (A) |
|  | Pull to reduce to A-10%=B, require continued pull for B? Yes=Positive LS |
|  | Do push test |
|  | Lateral/Directional Control Stability: Sideslip |
|  | Release Aileron (keep rudder), Do wings level? |
|  | # Static Directional Stability: Yaw w/ level wings, release rudder. Return to no yaw? |
|  | Spiral Stability: Bank 15 deg., release. Return to level? |
|  | ACCELERATED STALLS |
|  | 15 Degree bank, slow until stalls: Repeat w/ 30, 45, 60 |
|  | DESCENT RATE |
|  | Descend @ 90 MPH, record rate. Record loss of elevation w/ 90, 180, 360 degree turns |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

The objective is to incrementally test the affect of increased weight and aft weight on the aircraft handling.  DO NOT EXCEED GROSS WEIGHT.  STAY WITHING THE FORE/AFT CG LIMITS.

| Flight #: 12b – EXPLORE WEIGHT & BALANCE LIMITS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: Determine affect of change to aft balance and progressively increasing weights to establish maximum weight - Pilot (me) plus 160 lb. passenger & max. fuel |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Carefully weigh and secure balast |  |  |
|  | # Compute & record new weight & balance |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | Record climb performance: FPM = ____ |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | # SLOW FLIGHT & STALLS |  |  |
|  | 360 turns, slow flight & power off stall w/ no flaps, recover |  |  |
|  | Execute power-on stalls @ 2600 RPM |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | STABILITY & CONTROL CHECKS |
|  | # Longitudinal Stability: Record Airspeed @ 2800 RPM (A) |
|  | Pull to reduce to A-10%=B, require continued pull for B? Yes=Positive LS |
|  | Do push test |
|  | Lateral/Directional Control Stability: Sideslip |
|  | Release Aileron (keep rudder), Do wings level? |
|  | # Static Directional Stability: Yaw w/ level wings, release rudder. Return to no yaw? |
|  | Spiral Stability: Bank 15 deg., release. Return to level? |
|  | ACCELERATED STALLS |
|  | 15 Degree bank, slow until stalls: Repeat w/ 30, 45, 60 |
|  | DESCENT RATE |
|  | Descend @ 90 MPH, record rate. Record loss of elevation w/ 90, 180, 360 degree turns |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 12c – EXPLORE WEIGHT & BALANCE LIMITS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: Determine affect of change to aft balance and progressively increasing weights to establish maximum weight - Pilot (me) plus 200 lb. passenger & max. fuel |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Carefully weigh and secure balast |  |  |
|  | # Compute & record new weight & balance |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | Record climb performance: FPM = ____ |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | # SLOW FLIGHT & STALLS |  |  |
|  | 360 turns, slow flight & power off stall, recover |  |  |
|  | Execute power-on stalls @ 2600 RPM |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | STABILITY & CONTROL CHECKS |
|  | # Longitudinal Stability: Record Airspeed @ 2800 RPM (A) |
|  | Pull to reduce to A-10%=B, require continued pull for B? Yes=Positive LS |
|  | Do push test |
|  | Lateral/Directional Control Stability: Sideslip |
|  | Release Aileron (keep rudder), Do wings level? |
|  | # Static Directional Stability: Yaw w/ level wings, release rudder. Return to no yaw? |
|  | Spiral Stability: Bank 15 deg., release. Return to level? |
|  | ACCELERATED STALLS |
|  | 15 Degree bank, slow until stalls: Repeat w/ 30, 45 |
|  | DESCENT RATE |
|  | Descend @ 90 MPH, record rate. Record loss of elevation w/ 90, 180, 360 degree turns |
|  | # LANDING |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 12d – EXPLORE WEIGHT & BALANCE LIMITS |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: Determine affect of change to aft balance and progressively increasing weights to establish maximum weight - Pilot (me) plus 200 lb. passenger, max. fuel & 35 lbs. cargo |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Carefully weigh and secure ballast |  |  |
|  | # Compute & record new weight & balance |  |  |
|  | # DEPARTURE |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 6000’ MSL and level off |  |  |
|  | Record climb performance: FPM = ____ |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |
|  | # SLOW FLIGHT & STALLS |  |  |
|  | 360 turns, slow flight & power off stall, recover |  |  |
|  | Execute power-on stalls @ 2600 RPM |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | STABILITY & CONTROL CHECKS |
|  | # Longitudinal Stability: Record Airspeed @ 2800 RPM (A) |
|  | Pull to reduce to A-10%=B, require continued pull for B? Yes=Positive LS |
|  | Do push test |
|  | Lateral/Directional Control Stability: Sideslip |
|  | Release Aileron (keep rudder), Do wings level? |
|  | # Static Directional Stability: Yaw w/ level wings, release rudder. Return to no yaw? |
|  | Spiral Stability: Bank 15 deg., release. Return to level? |
|  | ACCELERATED STALLS |
|  | 15 Degree bank, slow until stalls: Repeat w/ 30, 45, 60 |
|  | DESCENT RATE |
|  | Descend @ 90 MPH, record rate. Record loss of elevation w/ 90, 180, 360 degree turns |
|  | # LANDING |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES

| Flight #: 13 – MAGNETIC COMPASS CALIBRATION |  |  |  |
| --- | --- | --- | --- |
| Pilot: |  | Date: | Time: |
| Objectives: - Adjust compass & calibrate compass card |  |  |  |
| Check | Action |  |  |
|  | # PRE-FLIGHT |  |  |
|  | Loosen compass adjustment cover |  |  |
|  | Make or buy a brass screwdriver |  |  |
|  | Pick a very calm morning |  |  |
|  | # DEPARTURE |  |  |
|  | Do not use flaps |  |  |
|  | Climb out at 110 MPH |  |  |
|  | Climb to 2000’ MSL and level off |  |  |
|  | # CRUISE |  |  |
|  | Limit prop RPM to 3200 |  |  |
|  | Check engine gauges especially CHT and Oil Temp |  |  |
|  | Trim hands off level flight |  |  |

NOTES

| Check | Action |
| --- | --- |
|  | CRUISE |
|  | # Fly North along a N/S Road |
|  | Check GPS compass heading |
|  | Adjust compass to read 360 degrees |
|  | Fly South, adjust compass |
|  | Fly East, adjust compass |
|  | # Fly West, adjust compass |
|  | Fly NW, record error ______ |
|  | Fly NE, record error ______ |
|  | Fly SE, record error ______ |
|  | Fly SW, record error ______ |
|  | Fly North, record error ______ |
|  | Fly South, record error ______ |
|  | Fly East, record error ______ |
|  | # Fly West, record error ______ |
|  | # LANDING |
|  | Use checklists |
|  | Fly pattern at 85 MPH |
|  | Taxi back and “Smile” |
|  | # POST FLIGHT |
|  | Record errors on compass card |
|  | Prepare corrective action list |
|  | Record fuel and oil consumption |

NOTES